# DMS HOTAS REFERENCE TABLE — COMPLETE MASTER MODE MATRIX

Prepared: 12 January 2026 | Source: Dash-34 Analysis | Purpose: Comprehensive quick-reference for all DMS operations

## TABLE 1: DMS VERTICAL (Up/Down) — BY MASTER MODE

### Master Mode: A-A (Air-to-Air)

| Parameter | Value | Notes |
| --- | --- | --- |
| DMS Up | ❌ INACTIVE | HUD never SOI in A-A mode |
| DMS Down | ✅ MFD ↔ MFD | Toggles SOI between Left and Right MFD |
| Permitted SOI Formats | FCR, HSD, TGP | Radar, tactical display, targeting pod only |
| Workflow | 1. Select MFD (Down) → 2. Cycle format (Left/Right) → 3. Designate (TMS+Cursor) | No HUD involvement |
| Visual Cue | SOI line around MFD border | Asterisk does NOT appear on HUD |

### Master Mode: DGFT (Dogfight)

| Parameter | Value | Notes |
| --- | --- | --- |
| DMS Up | ❌ INACTIVE | HUD never SOI in DGFT mode |
| DMS Down | ✅ MFD ↔ MFD | Toggles SOI between Left and Right MFD |
| Permitted SOI Formats | FCR, HSD, TGP | Radar, tactical display, targeting pod only |
| Workflow | 1. Select MFD (Down) → 2. TMS commands → 3. Cycle radar format (Left/Right) | High-G maneuvering; MFDs only |
| Visual Cue | SOI line around MFD border | Asterisk does NOT appear on HUD |

### Master Mode: MSL OVRD (Missile Override)

| Parameter | Value | Notes |
| --- | --- | --- |
| DMS Up | ❌ INACTIVE | HUD never SOI in MSL OVRD mode |
| DMS Down | ✅ MFD ↔ MFD | Toggles SOI between Left and Right MFD |
| Permitted SOI Formats | FCR, HSD, TGP | Radar, tactical display, targeting pod only |
| Workflow | 1. Select MFD (Down) → 2. Weapon page + SMS commands → 3. Fire | MFD-managed missile launch |
| Visual Cue | SOI line around MFD border | Asterisk does NOT appear on HUD |

### Master Mode: NAV (Navigation)

| Parameter | Value | Notes |
| --- | --- | --- |
| DMS Up | ✅ HUD → SOI | Designates HUD as SOI (heads-up navigation) |
| DMS Down (from HUD SOI) | ✅ HUD → MFD | Transfers SOI to last active MFD |
| DMS Down (from MFD SOI) | ✅ MFD ↔ MFD | Toggles SOI between Left and Right MFD |
| Permitted SOI Formats | HUD, FCR, TGP, WPN, HAD, HSD | Full sensor suite |
| Workflow | 1. DMS Up → HUD SOI → 2. Cursor to MARK point → 3. TMS Up → 4. DMS Down → MFD for detail | Heads-up route planning |
| Visual Cue | Asterisk (*) at upper-left of HUD when SOI | SOI line around MFD when MFD SOI |

### Master Mode: A-G (Air-to-Ground) — Preplanned Submodes (CCRP, LADD, DTOS)

| Parameter | Value | Notes |
| --- | --- | --- |
| DMS Up | ✅ HUD → SOI | Designates HUD as SOI (bomb-on-HUD delivery) |
| DMS Down (from HUD SOI) | ✅ HUD → MFD | Transfers SOI to last active MFD |
| DMS Down (from MFD SOI) | ✅ MFD ↔ MFD | Toggles SOI between Left and Right MFD |
| Permitted SOI Formats | HUD, FCR, TGP, WPN, HAD, HSD | Full sensor suite |
| Workflow | 1. DMS Up → HUD SOI → 2. TMS Up → crosshair to target → 3. TMS Down once locked → 4. Release when green cue | Heads-up CCIP/CCRP |
| Visual Cue | Asterisk (*) at upper-left of HUD when SOI | HUD shows bomb fall line + reticle |

### Master Mode: A-G (Air-to-Ground) — Visual Submodes (AGM-65 VIS, Visual CAS)

| Parameter | Value | Notes |
| --- | --- | --- |
| DMS Up | ✅ HUD → SOI | MANDATORY for visual targeting |
| DMS Down (from HUD SOI) | ⚙️ AUTO: HUD → WPN/TGP | Automatic after TMS Up designates target |
| DMS Down (manual) | ✅ Can toggle back to HUD | If pilot rejects and wants to re-designate |
| Permitted SOI Formats | HUD, TGP, WPN | Heads-up only; no preplanned sensors |
| Workflow | 1. DMS Up → HUD SOI [REQUIRED] → 2. Cursor + TMS Up → designate TV target → 3. Auto: SOI → WPN → 4. Slave seeker → 5. Fire | Visual weapons demand HUD |
| Visual Cue | Asterisk (*) on HUD for target designation | Auto-shifts to WPN page once locked |

### Master Mode: A-G (Air-to-Ground) — Bore Submode

| Parameter | Value | Notes |
| --- | --- | --- |
| DMS Up | ✅ HUD → SOI | Designates HUD as SOI (gun strafing, EEGS) |
| DMS Down (from HUD SOI) | ✅ HUD → MFD | Transfers SOI to last active MFD |
| DMS Down (from MFD SOI) | ✅ MFD ↔ MFD | Toggles SOI between Left and Right MFD |
| Permitted SOI Formats | HUD, WPN, FCR, TGP | Gun sight on HUD + weapon page info |
| Workflow | 1. DMS Up → HUD SOI → 2. Select gun (TMS Up/Down) → 3. Loft bomb or gun run | Strafing/gun-delivery focused |
| Visual Cue | Asterisk (*) on HUD gunsight | Crosshair + impact prediction |

### Master Mode: S-J (Selective Jettison)

| Parameter | Value | Notes |
| --- | --- | --- |
| DMS Up | ✅ HUD → SOI | Designates HUD as SOI |
| DMS Down (from HUD SOI) | ✅ HUD → MFD | Transfers SOI to last active MFD |
| DMS Down (from MFD SOI) | ✅ MFD ↔ MFD | Toggles SOI between Left and Right MFD |
| Permitted SOI Formats | HUD, FCR, SMS, HSD | Navigation + stores management |
| Workflow | 1. A-G mode → Select S-J on SMS page → 2. DMS Up/Down to manage stores display | Rarely used in combat |
| Visual Cue | SOI indicator same as A-G mode | Shows selected jettison group |

## TABLE 2: DMS HORIZONTAL (Left/Right) — FORMAT CYCLING

### Master Mode: A-A (Air-to-Air)

| Parameter | Value | Notes |
| --- | --- | --- |
| DMS Left | ✅ Cycles Left MFD | Primary → Sec1 → Sec2 → Primary (loop) |
| DMS Right | ✅ Cycles Right MFD | Primary → Sec1 → Sec2 → Primary (loop) |
| Available Formats | FCR (Primary), HSD, TGP, HAD, WPN | 5 major formats available |
| Blank Skipping | ✅ Auto-skip | Pilot never lands on BLANK page |
| Typical Sequence | FCR → HSD → TGP → FCR (loop) | Radar → tactical → targeting pod → repeat |
| Use Case | Switch radar mode mid-fight, check tactical for bandits, switch to TGP for gun support | Fast hands-on paging |

### Master Mode: DGFT (Dogfight)

| Parameter | Value | Notes |
| --- | --- | --- |
| DMS Left | ✅ Cycles Left MFD | Primary → Sec1 → Sec2 → Primary (loop) |
| DMS Right | ✅ Cycles Right MFD | Primary → Sec1 → Sec2 → Primary (loop) |
| Available Formats | FCR (Primary), HSD, TGP | Restricted to 3 key formats |
| Blank Skipping | ✅ Auto-skip | Unused formats are BLANK (skipped) |
| Typical Sequence | FCR → HSD → TGP → FCR (loop) | Radar search → tactical → targeting pod → repeat |
| Use Case | High-G turns; quickly check 6 o'clock (HSD), revert to radar, switch to TGP for close-in SRM | High workload situation |

### Master Mode: MSL OVRD (Missile Override)

| Parameter | Value | Notes |
| --- | --- | --- |
| DMS Left | ✅ Cycles Left MFD | Primary → Sec1 → Sec2 → Primary (loop) |
| DMS Right | ✅ Cycles Right MFD | Primary → Sec1 → Sec2 → Primary (loop) |
| Available Formats | FCR (Primary), HSD, TGP, WPN, SMS | 5 formats for missile management |
| Blank Skipping | ✅ Auto-skip | Pilot stays on useful pages |
| Typical Sequence | FCR → HSD → WPN → SMS → FCR | Radar → tactical → missile control → stores → repeat |
| Use Case | Cycle to WPN page to check missile status, SMS page to manage loadout, back to FCR for tracking | Hands-on weapon management |

### Master Mode: NAV (Navigation)

| Parameter | Value | Notes |
| --- | --- | --- |
| DMS Left | ✅ Cycles Left MFD | Primary → Sec1 → Sec2 → Primary (loop) |
| DMS Right | ✅ Cycles Right MFD | Primary → Sec1 → Sec2 → Primary (loop) |
| Available Formats | HSD (Primary), FCR, TGP, HAD, WPN, NAV | Full format suite available |
| Blank Skipping | ✅ Auto-skip | Unused formats are BLANK (skipped) |
| Typical Sequence | HSD → FCR → TGP → HSD (loop) | Tactical → radar → targeting pod → repeat |
| Use Case | Long-range cruise, check radar, review navigation POI, then revert to tactical display | Low-workload navigation |

### Master Mode: A-G Preplanned (CCRP, LADD, DTOS)

| Parameter | Value | Notes |
| --- | --- | --- |
| DMS Left | ✅ Cycles Left MFD | Primary → Sec1 → Sec2 → Primary (loop) |
| DMS Right | ✅ Cycles Right MFD | Primary → Sec1 → Sec2 → Primary (loop) |
| Available Formats | TGP (Primary), FCR, HSD, WPN, HAD, NAV | 6 formats for A-G targeting |
| Blank Skipping | ✅ Auto-skip | Only useful pages shown |
| Typical Sequence | TGP → FCR → WPN → TGP (loop) | Targeting pod → radar → weapons info → repeat |
| Use Case | Cycle to FCR for preplanned point correlation, WPN page for bomb info, back to TGP for fine targeting | Multimode bomb delivery |

### Master Mode: A-G Visual (AGM-65 VIS, CAS)

| Parameter | Value | Notes |
| --- | --- | --- |
| DMS Left | ✅ Cycles Left MFD | Primary → Sec1 → Sec2 → Primary (loop) |
| DMS Right | ✅ Cycles Right MFD | Primary → Sec1 → Sec2 → Primary (loop) |
| Available Formats | TGP (Primary), WPN | Minimal; visual weapons are simple |
| Blank Skipping | ✅ Auto-skip | Only TGP and WPN pages available |
| Typical Sequence | TGP → WPN → TGP (loop) | Camera → weapon control → camera |
| Use Case | Quick toggle between TGP video and seeker status (WPN page) during TV-guided missile flight | Real-time visual tracking |

### Master Mode: A-G Bore (Gun/EEGS)

| Parameter | Value | Notes |
| --- | --- | --- |
| DMS Left | ✅ Cycles Left MFD | Primary → Sec1 → Sec2 → Primary (loop) |
| DMS Right | ✅ Cycles Right MFD | Primary → Sec1 → Sec2 → Primary (loop) |
| Available Formats | WPN (Primary), TGP, HSD | 3 formats for gun/bore modes |
| Blank Skipping | ✅ Auto-skip | Unused formats are BLANK |
| Typical Sequence | WPN → TGP → WPN (loop) | Gun status → targeting pod → gun status |
| Use Case | Strafing with gun; check TGP for target image; revert to WPN for ammo/status | Gun-centric delivery |

### Master Mode: S-J (Selective Jettison)

| Parameter | Value | Notes |
| --- | --- | --- |
| DMS Left | ✅ Cycles Left MFD | Primary → Sec1 → Sec2 → Primary (loop) |
| DMS Right | ✅ Cycles Right MFD | Primary → Sec1 → Sec2 → Primary (loop) |
| Available Formats | SMS (Primary), FCR, HSD, NAV | 4 formats for stores management |
| Blank Skipping | ✅ Auto-skip | Pilot stays on valid pages |
| Typical Sequence | SMS → FCR → SMS (loop) | Stores management → tactical → repeat |
| Use Case | Emergency stores drop; cycle to SMS to select jettison group, check tactical before drop | Rare emergency mode |

## TABLE 3: CONSOLIDATED REFERENCE — ALL MASTER MODES

| Master Mode | DMS Up | DMS Down | DMS Left | DMS Right | SOI Formats | Key Constraint |
| --- | --- | --- | --- | --- | --- | --- |
| A-A | ❌ Block | MFD↔MFD | Cycle L | Cycle R | FCR, HSD, TGP | MFD-only SOI |
| DGFT | ❌ Block | MFD↔MFD | Cycle L | Cycle R | FCR, HSD, TGP | MFD-only SOI |
| MSL OVRD | ❌ Block | MFD↔MFD | Cycle L | Cycle R | FCR, HSD, TGP | MFD-only SOI |
| NAV | ✅ HUD | HUD↔MFD | Cycle L | Cycle R | HUD, FCR, TGP, WPN, HAD, HSD | HUD allowed |
| A-G (PRE) | ✅ HUD | HUD↔MFD | Cycle L | Cycle R | HUD, FCR, TGP, WPN, HAD, HSD | HUD allowed |
| A-G (VIS) | ✅ MANDATORY | Auto: HUD→WPN | Cycle L | Cycle R | HUD, TGP, WPN | Visual requires HUD |
| A-G (BORE) | ✅ HUD | HUD↔MFD | Cycle L | Cycle R | HUD, WPN, FCR, TGP | Gun/bore focused |
| S-J | ✅ HUD | HUD↔MFD | Cycle L | Cycle R | HUD, FCR, SMS, HSD | Jettison mode |

## TABLE 4: VISUAL CUES AND INDICATORS

| Indicator | Location | Meaning | Master Modes |
| --- | --- | --- | --- |
| **Asterisk (*)** | Upper-left HUD | HUD is SOI | NAV, A-G, S-J (when DMS Up active) |
| SOI Line | MFD border (perimeter) | MFD is SOI | All modes (when MFD is SOI) |
| Crosshair + "NOT SOI" | MFD (if not SOI) | Format inactive for targeting | All modes (on non-SOI formats) |
| HUD Blank/Inactive | HUD | HUD cannot be SOI | A-A, DGFT, MSL OVRD (DMS Up blocked) |

## TABLE 5: EXCEPTION RULES

| Condition | Result | DMS Behavior | Notes |
| --- | --- | --- | --- |
| MARK/OFLY Submode | SOI locked | DMS Up/Down blocked | Cannot change SOI during MARK designation |
| Snowplow (SP) PRE | SOI locked | DMS Up/Down blocked | Ground radar pre-tracking mode |
| MFD BIT (TEST) Mode | Formats duplicated | Temporary behavior | System performing self-test |
| AGM-65 VIS (auto handoff) | SOI→WPN auto | DMS Down doesn't interrupt | Seeker acquisition automatic |
| HARM HAS/POS | WPN SOI required | DMS must show WPN page | Anti-radiation targeting |
| Gun EEGS | HUD primary | DMS Up recommended | Gun gunsight on HUD essential |

## QUICK DECISION TREE

START: "What does DMS do in my mode?"

├─ Is it A-A / DGFT / MSL OVRD?
│  ├─ YES → Group A
│  │  ├─ DMS Up = BLOCKED (HUD not SOI)
│  │  ├─ DMS Down = Toggle MFD Left↔Right
│  │  └─ DMS Left/Right = Cycle formats (FCR/HSD/TGP)
│  │
│  └─ NO → Continue...
│
├─ Is it NAV / A-G Preplanned / Bore / S-J?
│  ├─ YES → Group B
│  │  ├─ DMS Up = HUD → SOI
│  │  ├─ DMS Down = Toggle HUD↔MFD or MFD↔MFD
│  │  └─ DMS Left/Right = Cycle formats (various)
│  │
│  └─ NO → Continue...
│
└─ Is it A-G Visual (AGM-65 VIS)?
   ├─ YES → Group C
   │  ├─ DMS Up = HUD → SOI (MANDATORY)
   │  ├─ DMS Down = Auto-transfer to WPN after TMS Up
   │  └─ DMS Left/Right = Cycle formats (TGP/WPN minimal set)
   │
   └─ ERROR: Unknown master mode

## FORMATTING NOTES FOR GUIDE

### How to Present Table 3 (Consolidated)

Option A: Single wide table (what you see here) - Pros: All data at once; good for laminated quick-reference cards - Cons: May be too wide for standard page layout

Option B: Split into 2 tables - Table 3a: DMS Up/Down behavior (columns: Master Mode, DMS Up, DMS Down, SOI Formats) - Table 3b: DMS Left/Right behavior (columns: Master Mode, DMS Left, DMS Right, Available Formats) - Pros: Better page fit; easier to read horizontally - Cons: Related data separated

Option C: Multiple mini-tables per section - Separate subsections (4.2 for Up/Down, 4.3 for Left/Right) - Each section has its own condensed table - Pros: Follows narrative flow; clearer grouping - Cons: Requires cross-referencing for full view

Recommendation: Option C (multiple mini-tables per section) — aligns with proposed chapter structure (4.2 Up/Down, 4.3 Left/Right).

## COMPLIANCE VERIFICATION

✅ Dash-34 Alignment - [x] DMS Up/Down rules match Section 2.1.1.2.3 (SOI mechanism) - [x] Master mode display formats match Section 2.1.1.2.1 (Master Mode Display Format table) - [x] SOI format restrictions match Section 2.1.1.2.3 (permitted displays)

✅ Logical Completeness - [x] All 8 master modes covered - [x] All 4 DMS directions covered (Up, Down, Left, Right) - [x] All SOI formats documented - [x] All visual cues explained

✅ Ready for Integration - [x] Can be split into Section 4.2 (Up/Down) + 4.3 (Left/Right) - [x] Can be merged into master HOTAS tables in later sections - [x] Serves as reference for WIP file development

Prepared by: AI Research Agent
Version: 1.0
Status: ✅ Ready for Author Review and WIP Integration
Date: 12 January 2026, 11:24 AM -03